# Copilot briefing — Solar Reflect report

Use this file as grounding when rewriting `report.docx`.

The report is a physics thought experiment: can an orbital specular mirror deliver useful sunlight to a dark site?

Reader: technically literate, not a specialist. Voice: conversational, short sentences, no lab jargon in the opening. Prefer mirror / sheet. Do not lead with foil. Do not say binds, vehicles, readout.

## How to help

Improve language, grammar, flow, and argument. Do not invent numbers, thresholds, figures, or physics.

User notes and suggested wording are a guide. If they clash with this briefing, this briefing wins.

Do not turn the piece into a company review, a product pitch, or a launch-cost study.

Do not conclude from things we did not model (clouds as peak I, light-curve integral, umbra along the pass, pointing error, diffraction, true midnight / polar summer, cost).

Civil night is the score. Terminator-nadir (looking straight down at dusk) is the brighter envelope, not the dark-city case. Do not mix them.

## The question (frozen)

A mirror of some size, at some height, with an optical factor for reflection and air. How bright is the ground patch? How wide? How long does it last at a site that is already dark (Sun 6° below the horizon)? Then: is that light useful compared with moonlight, street lighting, and sunlight for electricity? Those comparisons judge scale. They do not choose the physics.

## Master metaphor

Sun projector, not a spotlight. The mirror copies the Sun’s disk onto the ground. It does not make a tight lamp beam. Curving the mirror does not shrink that disk. Diffraction is negligible (Airy disk ~4 cm vs a ~6 km solar image at 625 km) and is not in the model.

55-metre square means 55 m on a side (area 3025 m²), building-sized. It is not 55 m².

## Two facts that cannot be engineered away

1. The Sun is a disk about half a degree wide (α = 0.0093 rad). Patch width at nadir: D = h α. At 625 km, D = 5.812 km, image area 26.53 km².

2. Low Earth orbit does not hover. Period at 625 km is 97.06 min. Useful window (elevation ≥ 30°) is 4.281 min overhead at dusk; 2.809 min at the civil-night snapshot. Unsteered flash across the site is 0.85 s. Area does not buy hours.

Brightness and duration are two currencies. A larger mirror buys brightness, not minutes. A longer pass (higher orbit) buys minutes and spends brightness. N = N_I × N_T for one site.

## Numbers you must not change (625 km)

Nadir envelope (twilight, looking down):

D = 5.812 km

M18 ideal I = 0.01175 W/m²; M55 ideal I = 0.1097 W/m²

T_useful = 4.281 min; horizon pass = 13.16 min

Fluence upper bound M55: ~28 J/m²

Civil night (the value case): sat still sunlit on the terminator, site Sun 6° down:

Path d = 937.5 km; elevation ε = 38.74°; bounce at mirror i = 67.63°

Ellipse ~8.7 × 14 km (D_minor = 8.719 km); A_image = 95.41 km²

I is 0.150 × the nadir envelope

M18 ideal I = 0.001759 W/m² (fails moonlight 0.003)

M55 ideal I = 0.01642 W/m² (moonlight pass; fails street 0.1)

1 km sheet: 5.429 W/m² (street-bright; fails 20 min — needs ~7.1 sequential passes)

T_useful = 2.809 min

400 km: no useful night pass (max elevation 27.2° < 30°)

Optical factor: η = 1 is ideal; 0.675 is one estimated clear-sky factor. Do not stack extra atmospheres.

Parked bins (scoring only): moonlight 0.003 W/m²; street-class 0.1 W/m² and 20 min; weak PV 50 W/m²; energy 200 W/m² and 3 h; daylight 1000 W/m².

Fleet, one site, η = 1, night: moonlight from M18 is N ≈ 1.7 (two mirrors). Street-class 20 min is N = 405 of 18 m, or ~43 of 55 m. If the mirror stays small, fewest street-class satellites on this scan are near 1500 km (M18: 405 → 196), not at 625 km.

GEO D ≈ 333 km — a size check, not an operating point.

## Figures in the report (do not add new ones)

Figure    Takeaway

|---|---|

S-geo-2d    The 45° bounce is at the mirror, not the ground. The patch is the Sun’s image.

S-focus    A dish focused at the ground does not shrink the solar image. Extra curve opens D.

S-spot-h    Width is height times how wide the Sun looks. 625 km → 5.812 km.

S-I-A    Peak I scales with collecting area over the solar image, not like a lamp 1/r².

S-dwell    Useful light is minutes. Area does not buy the window. Literature “~20 min at 1000 km” is the horizon pass (17.61 min), not useful time (6.727 min).

## Argument shape

Scene → flashlight picture → two facts → five corrections people reach for (spotlight / curve / bigger / hover / midnight) → two currencies → what the physics actually says → appendix numbers.

Midnight overhead in LEO is usually Earth’s shadow: nothing to bounce. You get early night from a still-sunlit terminator, at a penalty, or you get zero.

## Out of the report

Launch cost, firms, products, 3D explorer, lux as a kernel output, stacked suns past one filled image, one satellite for three hours, “streetlights from space” as the title subject.